ID :  LRUSWRS-1100

The LRU software shall repeat the CIP Configuration Discrete Test for a duration of up to 100 ms including the retry sequence prior to using Config_LRU_PSR as the next configuration source, if Config_CIP_Fault_Init is True after CIP completes execution.

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1101

The LRU software shall initialize Config_CIP_Fault_Init to False at the start of CIP.

Req Type :  Verbatim

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

Reference Requirements

ID :  LRUSWRS-1102

The LRU software shall read and debounce the Configuration Pins (Pin Functions ID0_DSP, ID1_DSP, ID2_DSP, ID3_DSP, ID4_DSP, ID5_DSP, ID6_DSP, ID7_DSP) for 10ms to compute Config_ID_Disc_Dbnce_Init.

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1103

The LRU software shall compute the Config_CIP_Fault_Init as follows:

if (SurfaceID == SURFC_INVALID)

Config_CIP_Fault_Init = TRUE

else

Config_CIP_Fault_Init = FALSE

Note:  The AssetType, Surface Location, and Parity_Disc_Dbnce_Init, will be checked by Surface ID, the SurfaceID = SURFC_INVALID if the debounced Configuration ID is not one of the entry in TABLE 1104 Rev C.

Req Type :  Decomposed

Safety Impact :  Yes

Safety Rationale :  This is the fault response for a safety monitor.

ID :  LRUSWRS-1104

Table 1104 Rev C  Asset Type & LRU Location Decode Based On Config_ID_Disc_Dbnce_Init

| ID7_DSP | ID6_DSP | ID5_DSP | ID4_DSP | ID3_DSP | ID2_DSP | ID1_DSP | ID0_DSP (Parity) | SurfaceID |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | TYPE_1_LRU_1 |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | TYPE_1_LRU_2 |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | TYPE_1_LRU_3 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | TYPE_1_LRU_4 |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | TYPE_1_LRU_5 |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | TYPE_1_LRU_6 |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | TYPE_1_LRU_7 |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | TYPE_2_LRU_1 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | TYPE_2_LRU_2 |
| 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | TYPE_2_LRU_3 |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | TYPE_2_LRU_4 |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | TYPE_2_LRU_5 |
| 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | TYPE_2_LRU_6 |
| 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | TYPE_2_LRU_7 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | TYPE_3_LRU_1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | TYPE_3_LRU_2 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | TYPE_3_LRU_3 |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | TYPE_3_LRU_4 |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | TYPE_3_LRU_5 |
| 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | TYPE_3_LRU_6 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | TYPE_3_LRU_7 |

Req Type :  Non-requirement

Safety Impact :  No

Safety Rationale :  Not a requirement

ID :  LRUSWRS-1104

The LRU software shall compute the Config_Location as follows:

if Config_CIP_Fault_Init is FALSE

Set Config_Location  = Config_Location_Disc

else if Corrupt_PSR_Rigging_CIP_Fault_Init is FALSE

Set Config_Location = Config_LRU_PSR

else

Set Config_Location = 0 (Invalid configuration)

Req Type :  Decomposed

Safety Impact :  Yes

Safety Rationale :  This is the fault response for a safety monitor.  One of the parent requirements have safety tags.