ID :  LRUSWRS-1200

After completion of the PIVT LRU Basic Test, the LRU software shall transmit the updated state of  LRU_Basic_PIVT_Fault to the LRU2 over the  STL Bus using IVT_Receipt_Envelope (0x0020) and IVT_Receipt_Content (0x000X).

where:

X=1 if LRU_Basic_PIVT_Fault is True

X=0 if LRU_Basic_PIVT_Fault False

Note: IVT_Receipt_Envelope and IVT_Receipt_Content holds the previously transmitted value until a new PIVT result is available for transmission to the LRU2. All IVT_Receipt_Envelope and IVT_Receipt_Content faults are cleared during power-up initialization, entry into PIVT Test Suite 2 and exit from IVT mode as commanded by IVT_Active_STL.

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1201

The LRU software shall repeat the RAM Memory test one time upon a RAM Memory Test failure.

Req Type :  Verbatim

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1202

The LRU software shall set RAM_Fault_GndOp to TRUE if the RAM Memory Test fails after the second attempt.

Req Type :  Verbatim

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1203

The LRU software shall send the IVT_Receipt_Envelope (0x0020) and IVT_Receipt_Content (0x0001) for any of the following conditions:

- RAM_Fault_GndOp is True

- Translation_Fault_GndOp is True

Note:  IVT_Receipt_Envelope (0x0020) and IVT_Receipt_Content (0x0001) is equivalent to LRU_Basic_PIVT_Fault = True

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1204

The LRU software shall send the IVT_Receipt_Envelope (0x0020) and IVT_Receipt_Content (0x0000) for the following conditions:

- RAM_Fault_GndOp is False AND

- Translation_Fault_GndOp is False

Note:  IVT_Receipt_Envelope (0x0020) and IVT_Receipt_Content (0x0000) is equivalent to LRU_Basic_PIVT_Fault = False

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1205

During the RAM Memory test, the LRU shall maintain the outputs state as follows:

- Valve_On is False

- Hi_Enable_Envelope is False

- Device_Charge_Order_Lim = 0.0

- STL_LRU_LRU2 is Enabled

Note:

-The STL_LRU_LRU2 is enabled upon completing initialization (See LRUSWRS-1786)

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1206

The RAM Memory Test shall write and read back the following test patterns:

- 0x55555555, 0xAAAAAAAA, and 0.

- walking address (i.e., address value is written into the address location inside each block)

e.g., for Block 0, 0x0009DFF9 is written into location 0x0009DFF9,

0x0009DFFA is written into location 0x0009DFFA,

0x0009DFFB is written into location 0x0009DFFB, …

for Block 1, 0x000BDDC0 is written into location 0x000BDDC0,

0x000BDDC1 is written into location 0x000BDDC1, ...

for Block 2, 0x000C1FF4 is written into location 0x000C1FF4, …)

to verify each RAM location in the blocks listed below.

NOTE: Because the test is invasive, the RAM test will exclude the OFP program memory.

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

Reference Requirements

ID :  LRUSWRS-1007

Table 1007: IVT Response and IVT Response Data definitions

| IVT_Receipt_Envelope | IVT_Receipt_Content | Response Description |
| --- | --- | --- |
| Word 10 | Word 11 |  |
| Message Byte Location 19, 18 | Message Byte Location 21, 20 |  |
|  |  | Test Suite 0 |
| 0011h | 000Xh | LRU Harmonizing Needed Test Completed with Results Bit 0: LRU_Harmonizing_Needed_PIVT 0=False, 1=True |
| 0012h | 00XXh | LRU Harmonizing Asset type and Location BIT 0-2: LRU Location BIT 3-4: LRU Asset type |
| 0013h | XXXXh | LRU Harmonizing Asset Serial Number BIT 0-15: LRU Asset Serial Number |
| 0014h | XXXXh | BIT 0-15: LRU Harmonizing Gain |
| 0015h | XXXXh | BIT 0-15: LRU Harmonizing Offset |
| 0016h | 000Xh | LRU Auto Harmonizing Results BIT 0: LRU_AutoHarmonize_Pass_PIVT 0=False, 1=True BIT 1: LRU_AutoHarmonize_Fail_PIVT 0=False, 1=True |
| 0017h | 00XXh | LRU Electronic Harmonizing Results BIT 0: Contract_Stop_Collection_Complete 0=False, 1=True BIT 1: Expand_Stop_Collection_Complete 0=False, 1=True BIT 2: VOID_Collection_Complete 0=False, 1=True BIT 3: Harmonizing_Complete 0=False, 1=True BIT 4: Harmonizing_Failed 0=False, 1=True |
| 0018h | XXXXh | LRU Upper Limit BIT 0-15: LRU Upper Limit |
| 0019h | XXXXh | LRU Lower Limit BIT 0-15: LRU Lower Limit |
|  |  | Test Suite 2 |
| 0020h | 000Xh | LRU Core Test Completed with Results BIT 0: LRU_Core_PIVT_Fault 0=False, 1=True |
| 0021h | 000Xh | LRU Driver Test Complete with Results BIT 0: Driver_PIVT_Fault_Code, 0=False, 1=True |
| 0022h | 000Xh | LRU Connect Test Completed with Results BIT 0, Connect_PIVT_Fault_Code, 0=False, 1=True |
| 0024h | 000Xh | LRU Solenoid Ready Status BIT 0: 1 = LRU Solenoid Ready |

Req Type :  Non-requirement

Safety Impact :  No

Safety Rationale :  Not a requirement.

ID :  LRUSWRS-1008

Table 1008, Rev B-  IVT Command and IVT Command Data definition

| IVT_Command_ISM | IVT_CmdData_ISM | Signal Definition | Command Description |
| --- | --- | --- | --- |
| STL Word 7 | STL Word 8 |  |  |
| Message Byte 15,14 | Message Byte 17,16 (Hexadecimal Representation) |  |  |
| 0x0011 | 0000 | Start_PIVT_Test_Suite_0_STL | Start PIVT Test Suite 0 |
| 0x0012 | 0001 | Harmonizing_and_Configuration_Parameter_Request_STL | Harmonizing and Configuration Parameter Request |
| 0x0013 | 000X | LRU_Auto_Harmonize_Available_STL | LRU AutoHarmonizegng Possible BIT 0: LRU_Auto-Harmonize_Available 0=False, 1=True |
| 0x0014 | 00XX | LRU_2_Config_STL | LRU Harmonizing Location and Asset type BIT 0-2 LRU Location BIT 3-4 Asset type |
| 0x0015 | XXXX | LRU_2_Asset_ID_STL | LRU Harmonizing Asset Serial Number BIT 0-15: LRU Asset ID |
| 0x0016 | XXXX | LRU_Harmonizing_Gain_STL | LRU Harmonizing Data BIT 0-15: LRU Harmonizing Gain |
| 0x0017 | XXXX | LRU_Harmonizing_Offset_STL | LRU Harmonizing Data BIT 0-15: LRU Harmonizing Offset |
| 0x0018 | XXXX | LRU_2_Act_Disp_Raw_Avg_Contract_STL | LRU_2 Harmonizing Data BIT 0-15: Average Contract Position |
| 0x0019 | XXXX | LRU_2_Act_Disp_Raw_Avg_Expand_STL | LRU_2 Harmonizing Data BIT 0-15: Average Expand Position |
| 0x001A | XXXX | Upper_Limit_LRU_STL | LRU Upper Limit |
| 0x001B | XXXX | Lower_Limit_LRU_STL | LRU Lower Limit |
| 0x0020 | 000X BIT 1 | Harmonize_Contract_Stop_STL | Start LRU Electronic Harmonizing at Contract stop position |
| 0x0020 | 000X BIT 2 | Harmonize_Expand_Stop_STL | Start LRU Electronic Harmonizing at Expand stop position |
| 0x0020 | 000X BIT 3 | Harmonize_VOID_STL | Start LRU Electronic Harmonizing at VOID position |
| 0x0021 | 0000 | Start_PIVT_Test_Suite_2_STL | Start PIVT Test Suite 2 |
| 0x0022 | 0000 | Engage_LRU_MSV_SOV_STL | Engage LRU MSV SOV |
| 0x0030 | 0000 | LRU_Match_Index_Request_STL | Request LRU SW Match Valie |
| 0x0040 | 0000 | Err_Log_Request_STL | Request Transfer of LRU Error History |

Req Type :  Non-requirement

Safety Impact :  No

Safety Rationale :  Not a requirement