ID :  LRUSWRS-1400

The LRU shall set Low_Threshold_Anomaly_NodeB_Monitor to True if any of the following signals are True:

- Rotor_Ref_Bad_Monitor

- Pre_Drive_Supply_Bad_Monitor

And False otherwise.

Note: Rotor_Ref_Bad_Monitor and Pre_Drive_Supply_Bad_Monitor are contained within the FPGA_FRAME_SYNC_REG as defined in Table 1401

Req Type :  Verbatim

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1402

The LRU shall delay the rising edge transition of Hi_Enable_Envelope

to yield Hi_Enable_Delay_Envelope with a delay of 4 +/- 2 msec.

Req Type :  Verbatim

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1403

The LRU shall delay the rising edge transition of Valve_On

to yield Valve_On_Delay with a delay of 4 +/- 2 msec.

Req Type :  Verbatim

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1404

The LRU software shall compute the Speed_Loop_Clear flag as follow:

if (Low_Threshold_Anomaly_NodeB_Monitor = TRUE OR Hi_Enable_Delay_Envelope = FALSE)

Then

Set Speed_Loop_Clear = TRUE

else

Set Speed_Loop_Clear = FALSE

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

Reference Requirements

ID :  LRUSWRS-1405

While in Running Mode, the LRU shall transition from Idle State to Speed State on Primary and Spoiler surfaces if all of the following conditions are met:

- Elec_En_STL is equal to TRUE

- Rect_ok_Dbnce_Lgc is equal to TRUE

- STL_Bad_Message is equal to FALSE

- Elec_Bad_Monitor_LRU is equal to FALSE.

Req Type :  Verbatim

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1406

The LRU software shall set the Hi_Enable_Envelope to TRUE while in the Speed_State_StateLgc.

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1407

The LRU software shall transition from Idle_Mode_StateLgc to Location_State_StateLgc on primary surfaces if all the following conditions are met:

- Bus_OK_Message is equal to TRUE

- Rect_ok_Dbnce_Lgc is equal to FALSE

- Hyd_ok_Dbnce_Lgc is equal to FALSE

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1408

The LRU software shall energize the MSV solenoid for primary surfaces (LRU_1, LRU_2, LRU_5, LRU_6, LRU_7) while in Location_State_StateLgc by setting the following bits:

- Valve_On to TRUE using the FPGA register DSP_Control register bit in Table 1401.

- Valve_On discrete IO to TRUE using the VALVE_ENABLE_DSP Pin function in Table 1409.

Req Type :  Decomposed

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.

ID :  LRUSWRS-1401

Table 1401, Rev B  - FPGA Register Interface

| Offset | SW Read/Write Register Access | Register Type/Bit Location | Register/Bit Signal Name | Register/Bit Description | Scale Factor | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00 | FPGA_0x00_REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Information01 | Information_Description | N/A | N/A |
| 0x01 | FPGA_0x01_REG |  |  |  |  |  |
|  | W / R | 8 Bit Vector | Information11 | Information_Description | N/A | N/A |
|  |  | Bit 0 | Information12 | Information_Description | N/A | [0-7] |
|  |  | Bit 1 | Information13 |  |  |  |
|  |  | Bit 2 | Information14 |  |  |  |
|  |  | Bits 3 - 7 | Information15 | Information_Description | N/A | N/A |
| 0x02 | FPGA_0x02_REG |  |  |  |  | Information |
|  | R | 16 Bit Vector | Information21 | Information_Description | N/A | N/A |
|  |  | Bit 0 | Information22 | Information_Description | N/A | [0,1] |
|  |  | Bit 1 | Information23 | Information_Description | N/A | [0,1] |
|  |  | Bit 2 | Information24 | Information_Description | N/A | [0,1] |
|  |  | Bit 3 | Information25 | Information_Description | N/A | [0,1] |
|  |  | Bit 4 | Information26 | Information_Description | N/A | [0,1] |
|  |  | Bit 5 | Information27 | Information_Description | N/A | [0,1] |
|  |  | Bit 6 | Information28 | Information_Description | N/A | [0,1] |
|  |  | Bit 7 | Information29 | Information_Description | N/A | [0,1] |
|  |  | Bit 8 | Information211 | Information_Description | N/A | [0,1] |
|  |  | Bit 9 | Information212 | Information_Description | N/A | [0,1] |
|  |  | Bit 10 | Information213 | Information_Description | N/A | [0,1] |
|  |  | Bit 11 | Information214 | Information_Description | N/A | [0,1] |
|  |  | Bit 12 | Information215 | Information_Description | N/A | [0,1] |
|  |  | Bits 13-14 | Information216 | Information_Description | N/A | [0-3] |
|  |  | Bit 15 | Information217 | Information_Description | N/A | [0,1] |
| 0x03 | FPGA_0x03_REG |  |  |  |  |  |
|  | R / W | 16 Bit Unsigned | Underlying_Layer_Test | The Underlying Layer SBIT Test fails if Underlying_Layer_Monitor_Init is True |  |  |
| 0x05 | FPGA_0x05_REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Layer_2_Match_Info | The Layer 2 index for use by the SBIT Layer 1 – Layer 2 Compatibility test, initial value is 1. | N/A | [1, 65535] |

Table 1401, Rev B - FPGA Register Interface (Continued)

| Offset | SW Read/Write Register Access | Register Type/Bit Location | Register/Bit Signal Name | Register/Bit Description | Scale Factor | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| 0x0A | FPGA_DSP_FLAGS_REG |  |  |  |  |  |
|  | R / W | 8 Bit Vector | DSP_Control | The DSP control register commands the FPGA states and corresponding output drive signals | N/A | N/A |
|  |  | Bit 0 | Valve_On | 1 = DSP commands Solenoid on | N/A | [0,1] |
|  |  | Bit 1 | Spare | Spare | N/A | N/A |
|  |  | Bit 2 | Location_State_StateLgc | 1 = EBMCE is in Position State | N/A | [0,1] |
|  |  | Bit 3 | Speed_State_StateLgc | 1 = EBMCE is in Velocity State | N/A | [0,1] |
|  |  | Bit 4 | Backup_Control_Active_LA_StateLgc | 1 = Back-Up Control is in control | N/A | [0,1] |
|  |  | Bits 5 – 7 | Spare | Spare | N/A | N/A |
| 0x0B | FPGA_0x0B_REG |  |  |  |  |  |
|  | R / W | 12 Bit Vector | InformationB1 | Information_Description | N/A | N/A |
|  |  | Bit 0 | InformationB2 | Information_Description | N/A |  |
|  |  | Bit 1 | InformationB3 | Information_Description | N/A | [0,1] |
|  |  | Bit 2 | InformationB4 | Information_Description | N/A | [0,1] |
|  |  | Bit 3 | InformationB5 | Information_Description | N/A | [0,1] |
|  |  | Bit 4 | InformationB6 | Information_Description | N/A | [0,1] |
|  |  | Bit 5 | InformationB7 | Information_Description | N/A | [0,1] |
|  |  | Bit 6 | InformationB8 | Information_Description | N/A | [0,1] |
|  |  | Bit 7 | InformationB9 | Information_Description | N/A | [0,1] |
|  |  | Bit 8 | InformationB21 | Information_Description | N/A | [0,1] |
|  |  | Bit 9 | InformationB22 | Information_Description | N/A | [0,1] |
|  |  | Bit 10 | InformationB23 | Information_Description | N/A | [0,1] |
|  |  | Bit 11 | InformationB24 | Information_Description | N/A | [0,1] |
| 0x0C | FPGA_0X0C_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Device_Position_Meas_Return | Device Location Info | LSB = 360/4096 = 0.087890625 degrees | [0, 4095] => 0 - 360 degrees |
| 0x0E | FPGA_0X0E_REG |  |  |  |  |  |
|  | R | 16 Bit Signed | Device_Speed_Err_Info | Device Speed Error | LSB = 29.296875 rpm/bit | [-32,768, 32,767] => ±60,000 rpm. |

Table 1401, Rev B - FPGA Register Interface (Continued)

| Offset | SW Read/Write Register Access | Register Type/Bit Location | Register/Bit Signal Name | Register/Bit Description | Scale Factor | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| 0x20 | FPGA_0X20_REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Information201 | Information_Description | N/A | N/A |
| 0x21 | FPGA_0X21_REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Information211 | Information_Description | N/A | N/A |
| 0x24 | FPGA_0X24_REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Information241 | Information_Description | N/A | [0-7] |
| 0x30 | FPGA_0X30_REG |  |  |  |  |  |
|  | R / W | 12 Bit Signed | Information301 | Information_Description | LSB=0.101612 | [-2048, 2047] |
| 0x31 | FPGA_0X31_REG |  |  |  |  |  |
|  | R / W | 12 Bit Signed | Information311 | Information_Description | LSB=0.025 V | [-2048, 2047] |
| 0x40 | FPGA_0X40_REG |  |  |  |  |  |
|  | R / W | 12 Bit Unsigned | Information401 | Information_Description | LSB = 0.001221 V | [0,4095] => 0-5 V Saturation after 5 V. |
| 0x41 | FPGA_0X41_REG |  |  |  |  |  |
|  | R / W | 12 Bit Unsigned | Information411 | Information_Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x42 | FPGA_0X42_REG |  |  |  |  |  |
|  | R / W | 12 Bit Unsigned | Information421 | Information_Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x44 | FPGA_FRAME_SYNC_REG |  |  |  |  |  |
|  | R | 16 Bit Vector | Information441 | Information_Description | N/A | N/A |
|  |  | Bit 0 | Information442 | Information_Description | N/A | N/A |
|  |  | Bit 1 | Information443 | Information_Description | N/A | [0,1] |
|  |  | Bits 2 - 3 | Information444 | Spare | N/A | N/A |
|  |  | Bit 4 | Power_NA_Err | 1 = External 28 VDC power is below minimal threshold | N/A | [0,1] |
|  |  | Bit 5 | Information446 | Information_Description | N/A | [0,1] |
|  |  | Bits 6 – 8 | Information447 | Spare | N/A | N/A |
|  |  | Bit 9 | Information448 | Information_Description | N/A | [0-7] |
|  |  | Bit 10 | Information449 | Information_Description | N/A |  |
|  |  | Bit 11 | Information450 | Information_Description | N/A |  |
|  |  | Bits 12 - 13 | Information451 | Spare | N/A | N/A |
|  |  | Bit 14 | Rotor_Ref_Bad_Monitor | Rotor Ref Bad Monitor | N/A | [0,1] |
|  |  | Bit 15 | Pre_Drive_Supply_Bad_Monitor | Pre Drive Supply Bad Monitor | N/A | [0,1] |

Table 1401, Rev B - FPGA Register Interface (Continued)

| Offset | SW Read/Write Register Access | Register Type/Bit Location | Register/Bit Signal Name | Register/Bit Description | Scale Factor | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| 0x61 | FPGA_0X61_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Temp_Meas_Analog611 | Board Temperature | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x63 | FPGA_0X63_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Volt_Analog631 | Voltage Info for A/D | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x64 | FPGA_0X64_REG |  |  |  |  |  |
|  | R | 12 Bit Signed | Test_ADC641 | Test ADC Register | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x65 | FPGA_0X65_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Solenoid_Volt_Meas_Analog651 | MSV Drive Feedback Voltage | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x66 | FPGA_0X66_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Volt_Ground_1_Analog661 | Ground Reference voltage 1 | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x67 | FPGA_0X67_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Volt_Ground_2_Analog671 | Ground Reference voltage 2 | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x68 | FPGA_0X68REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Bar_Transformer_A681 | Transformer A Input | N/A | [0, 65535] |
| 0x69 | FPGA_0X69_FB |  |  |  |  |  |
|  | R | 16 Bit Signed | Information691 | Information Description | LSB=0.101612 V | [-2048, 2047] => ±208A |
| 0x6A | FPGA_0X6A_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information6A1 | Information Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x6B | FPGA_0X6B_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information6B1 | Information Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x6C | FPGA_0X6C_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information6C1 | Information Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x6D | FPGA_0X6D_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information6D1 | Information Description | LSB = 0.001221 V | [0, 4095] => 0~5V |

Table 1401, Rev B - FPGA  Register Interface (Continued)

| Offset | SW Read/Write Register Access | Register Type/Bit Location | Register/Bit Signal Name | Register/Bit Description | Scale Factor | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| 0x70 | FPGA_0X70_REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Information701 | Information Description | LSB = 0.0382418 Amps | [0, 4095] => 0~156.6A Note: Sign is extended from 12 to 16 bits. There is no negative current range. |
| 0x71 | FPGA_0X71_REG |  |  |  |  |  |
|  | R | 16 Bit Signed | Information711 | PBIT Motor Information Description | LSB = 0.024426 V | [-2048, 2047] => ±50V |
| 0x72 | FPGA_0X72_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information721 | Information Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x73 | FPGA_0X73_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information731 | Information Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x74 | FPGA_0X74_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information741 | Information Description | LSB = 0.001221 V | [0, 4095] => 0~5V |
| 0x75 | FPGA_0X75_REG |  |  |  |  |  |
|  | R | 12 Bit Unsigned | Information751 | Information Description | LSB = 0.023443 V | [0, 4095] => 0~96V |
| 0x78 | FPGA_0X78_REG |  |  |  |  |  |
|  | R | 16 Bit Unsigned | Information781 | Information Description | N/A | [0, 65535] |

Table 1401, Rev B - FPGA Register Interface (Continued)

| Offset | SW Read/Write Register Access | Register Type/Bit Location | Register/Bit Signal Name | Register/Bit Description | Scale Factor | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| 0x80 | R | 16 Bit Unsigned | Information801 | Information Description | 12 bit LSB = 0.08791209 degrees 16 bit LSB = 0.00549 degrees. | [0, 4095] => 0 - 360 mechanical motor degrees |
| 0x82 | R | 16 Bit Signed | Information821 | Information Description | LSB = 29.28 rpm | [-2048, 2047] => ±60,000 rpm |
| 0x88 | R / W | 8 bit Unsigned | Information881 | Information Description | 12 bit LSB = 0.08833 degrees 16 bit LSB = 0.00500 degrees | [0, 255] Default = 2.2 V [0 – 4.82] V (functional range) |
| 0x89 | R / W | 8 Bit Unsigned | Information891 | Information Description | LSB = 38 mV | [0, 255] Default = 4.1 V |
| 0x8A | R / W | 8 Bit Unsigned | Information8A1 | Information Description | LSB = 38 mV | [0, 255] Default = 380 mV |
| 0x8B | R / W | 8 Bit Unsigned | Information8B1 | Information Description | LSB = 38 mV | [0, 255] |
| 0x8C | R / W | 8 Bit Unsigned | Information8C1 | Information Description | LSB = 38 mV | [0, 255] |
| 0x8D | R / W | 8 Bit Unsigned | Information8D1 | Information Description | 12 bit LSB = 0.14 degrees 14 and 16 bit LSB- = 0.09 degrees | [0, 255] 12 bit resolution: Default = 5.0 degrees 14 and 16 bit resolution: Default = 2.5 degrees |
| 0x8E | R / W | 8 Bit Unsigned | Information8E1 | Information Description | 12 bit LSB = 0.14 degrees 14 and 16 bit LSB- = 0.09 degrees | [0, 255] 12 bit resolution: Default = 1.0 degrees 14 and 16 bit resolution: Default = 0.5 degrees |

Table 1401, Rev B - FPGA Register Interface (Continued)

| Offset | SW Read/Write Register Access | Register Type/Bit Location | Register/Bit Signal Name | Register/Bit Description | Scale Factor | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| 0x91 | R / W | 8 Bit Unsigned | Information911 | Information Description | LSB (Hz) =3.05176 x 10^-5 x clock_freq | [0, 255] Default = 0x41 |
| 0x92 | R / W | 8 Bit Unsigned | Information921 | Information Description | N/A | [0, 255] Default = 0x7E |
| 0xF0 | W | 8 Bit Unsigned | InformationF01 | Information Description | N/A | N/A |
| 0xFF | R | 8 Bit Unsigned | InformationFF1 | - Information Description | N/A | [0, 255] |

Req Type :  Non-requirement

Safety Impact :  No

Safety Rationale :  Not a requirement.

ID :  LRUSWRS-1409

The LRU software shall map the discrete input/output functions per the following pin configurations:

Table 1409 Pin Configurations

| Pin Name | Pin Number | Pin Function | Description |
| --- | --- | --- | --- |
| DAI1 | A8 | ID0_DSP | Config_ID_Parity |
| DAI2 | B9 | ID1_DSP | Config_ID_1_Disc |
| DAI3 | B8 | ID2_DSP | Config_ID_2_Disc |
| DAI4 | A7 | ID3_DSP | Config_ID_3_Disc |
| DAI5 | B1 | ID4_DSP | Config_ID_4_Disc |
| DAI6 | D2 | ID5_DSP | Config_ID_5_Disc |
| DAI7 | C2 | ID6_DSP | Config_ID_6_Disc |
| DAI8 | E2 | ID7_DSP | Config_ID_7_Disc |
| DAI11 | E1 | RECT1_OK_DSP | Rect_OK_Disc |
| DAI12 | F2 | HYd2_OK_DSP | Hyd_OK_Disc |
| DAI15 | G1 | Hi_ENABLE_DSP_NOT | Hi_Enable_Envelope |
| DAI16 | H2 | VALVE_ENABLE_DSP | Valve_On |

Req Type :  Derived

Safety Impact :  No

Safety Rationale :  This is a detailed implementation requirement.